- Using the "Character Count" framing method, decode the following stream of bits into complete frames.

- Bit Stream (represented as hexadecimal): 03 CA 93 04 30 01 3C

- In a communication system with “Flag with Bit Stuffing” applied for framing, if following data bits are received, please deduce the original message bits.

0011 1110 0111 1100 0011 1110 0011 1000

- In a Hamming code system (m = 7, r = 3), the received codeword (left to right as position 1 to 7) is 1011010. Is this a correct codeword? Please correct it, if it is incorrect.

- A sender uses the CRC (Cyclic Redundancy Check) method with the generator polynomial G(x) = x³ + x + 1 for error detection. The original data bits to be sent are 10110. What is the final transmitted bit sequence (data + CRC bits)?

- For go-back-n protocol, if 8 bits are used for representing sequence number, 1) what will be the maximum limit of sending window size? 2) with the same sequence number length, if selective-repeat protocol is used, what will be the maximum receiving window size?