- Please use Manchester Encoding and Differential Manchester Encoding for representing a bit string: 1010

- If the following frame arrives at bridge X’s interface X11, please explain how it will be forwarded and how bridge X will update its MAC table.(5 marks)

- Frame:

| - ... | - 0001-0001-0001 | - 0002-0002-0002 | - ... | - ... |
| --- | --- | --- | --- | --- |
| - ... | - Source Address | - Destination Address | - Payload data | - ... |

- MAC table:

| - MAC Address | - Port/Interface |
| --- | --- |
| - 0003-0003-0003 | - X12 |
| - 0004-0004-0004 | - X13 |

- A CSMA/CD network has: Bandwidth: 200 Mbps; Maximum distance: 100 meters; Signal speed: 200 m/μs. What is the minimum frame length required?

- Please use Dijkstra to compute a sink tree consisting all the shortest paths rooted at node A?

- Router C receives distance vectors from neighbors B, D, and E :

- ​​From B: (A:5, B:0, C:8, D:12, E:6, F:2)

- ​​From D: (A:16, B:12, C:6, D:0, E:9, F:10)

- ​​From E: (A:7, B:6, C:3, D:9, E:0, F:4)

- The delays from C to its neighbors D, D and E are measured as: C→B: 6; C→D: 3; C→E: 5. 	Please update Router C’s distance vector.​​

- For an IP address 172.16.20.200, please tell which Class it belongs to, and what is its corresponding network address and broadcast address.

- For a network IP 202.38.197.0, if 2 bits are borrowed as subnet ID, how many subnets can be created? And totally how many valid host IP addresses can be supported?

- An Internet Service Provider (ISP) has been allocated the address block 172.20.0.0/16.A customer requires the following three networks, with the number of hosts indicated:

- Network X – 2000 hosts

- Network Y – 1000 hosts

- Network Z – 4000 hosts

- Using CIDR/VLSM to save address space, allocate subnets from the 172.20.0.0/16 block.

- List the network address and subnet mask for each network (X, Y, Z).